# Manufacturer: Jacksons Fencing

# Web: www.jacksons-security.co.uk

# Tel: +44 (0)1233 750393 | 0800 953 3699

# Product Reference: Barbican® A1 (SR1)

LPS 1175 A1 certified square pale vertical bar fencing

# Guarantee:

Steel fencing and gates designed and manufactured to be fit for purpose and providing a minimum 25 year guarantee, delivering the lowest whole life cost and significantly reduced carbon footprint.

# Fencing & Posts:

[1800mm High, post dimensions 60 x 60mm, pales cut at 45 degrees to form a point]

[2000mm High, post dimensions  60 x 60mm, pales cut at 45 degrees to form a point]

[2500mm High, post dimensions  80 x 60mm, pales cut at 45 degrees to form a point]

[3000mm High, post dimensions 80 x 60mm, pales cut at 45 degrees to form a point]

[Other heights between 1800-3000mm available, please specify]

# Panel Construction:

25 x 25mm rectangular hollow section pales set diamond to view and passed through the top and bottom rails with spacing at 129mm centres (96mm gap), and welded into position. Cut at a 45 degree angle.

Post Construction:

Posts are made using rectangular hollow section steel hot dip galvanised inside and out to BS EN 1461 as standard and supplied with flat post caps in black polypropylene.

# Fixings:

Fence panels are fixed to posts with matching Jacksons proprietary concealed vandal-proof panel fixings

# Finish / Colour:

[Fence panels and posts supplied galvanised inside and out to BS EN 1461 as standard]

[Fence panels and Posts galvanised inside and out to BS EN 1461 then polyester powder coated with Akzo Nobel Interpon coating in standard [Black RAL 9005]. [Green RAL 6005].  [White RAL 9010]. [Blue RAL 5010]. [Blue BS 20-C-40]. [Brown RAL 8017]. [Yellow RAL 1018]. [Grey RAL 7012]. [Red RAL 3020]. Or special [Insert RAL/BS].

For installations within 500m of salt water or estuary [Marine coated Insert RAL/BS]

# Installation:

# Set out and erect:

- Alignment: Straight lines or smoothly flowing curves, fence line may be stepped or raked to accommodate sloping or uneven ground.

- Tops of posts: Following profile of the ground.

- Setting posts: Rigid, plumb and to specified depth, or greater where necessary to ensure adequate support.

- Fixings: All components securely fixed.

# Setting posts in concrete:

- Standard: To BS 8500-2.

- [Mix: Designated concrete not less than GEN1 or Standard prescribed concrete not less than ST2.]

- [Alternative mix for small quantities: 50kg Portland cement to150kg fine aggregate to 250kg, 20mm nominal maximum size coarse aggregate, medium workability.]

- [Admixtures: Do not use].

- Holes: Excavate neatly and with vertical sides.

- Filling: Position post/strut and fill hole with concrete to not less than the specified depth, well rammed as filling proceeds and consolidated.

- [Backfilling of holes not completely filled with concrete: Excavated material, well rammed and consolidated.

# Completion:

# Cleaning:

- General: Leave the works in a clean, tidy condition.

- Surfaces: Clean immediately before handover.

# Fixings:

- All components: Tighten.

- Timing: Before handover.