# Manufacturer: Jacksons Fencing

# Web: www.jacksons-security.co.uk

# Tel: +44 (0)1233 750393 | 0800 953 3699

# Product Reference: Barbican Imperial A1 (SR1)

LPS 1175 A1 certified rounded pale vertical bar railings

# Guarantee:

Steel fencing and gates designed and manufactured to be fit for purpose and providing a minimum 25 year guarantee, delivering the lowest whole life cost and significantly reduced carbon footprint.

# Fencing & Posts:

[1800mm High, post dimensions 60 x 60mm, pales cut at 45 degrees to form a point]

[2000mm High, post dimensions  60 x 60mm, pales cut at 45 degrees to form a point]

[2500mm High, post dimensions  80 x 60mm, pales cut at 45 degrees to form a point]

[3000mm High, post dimensions 80 x 60mm, pales cut at 45 degrees to form a point]

[Other heights between 1800-3000mm available, please specify]

# Panel Construction:

1800-2000mm:

Dia 25mm round hollow section pales are passed through the top and bottom rails with spacing at 124mm centres (99mm gap) (which complies with Building Regulations), and welded into position . Pales protrude 175mm above the top rail and 100mm below the bottom rail.

2500-3000mm:

Dia 32mm round hollow section pales are passed through the top and bottom rails with spacing at 129mm centres (97mm gap) (which complies with Building Regulations, and welded into position. Pales protrude 175mm above the top rail and 100mm below the bottom rail.

# Post Construction:

Posts are made using rectangular hollow section steel hot dip galvanised inside and out to BS EN 1461 as standard and supplied with flat post caps in black polypropylene.

# Fixings:

Fence panels are fixed to posts with matching Jacksons proprietary concealed vandal-proof panel fixings

# Finish / Colour:

[Fence panels and posts supplied galvanised inside and out to BS EN 1461 as standard]

[Fence panels and Posts galvanised inside and out to BS EN 1461 then polyester powder coated with Akzo Nobel Interpon coating in standard [Black RAL 9005]. [Green RAL 6005].  [White RAL 9010]. [Blue RAL 5010]. [Blue BS 20-C-40]. [Brown RAL 8017]. [Yellow RAL 1018]. [Grey RAL 7012]. [Red RAL 3020]. Or special [Insert RAL/BS].

For installations within 500m of salt water or estuary [Marine coated Insert RAL/BS]

# Installation:

# Set out and erect:

- Alignment: Straight lines or smoothly flowing curves, fence line may be stepped or raked to accommodate sloping or uneven ground.

- Tops of posts: Following profile of the ground.

- Setting posts: Rigid, plumb and to specified depth, or greater where necessary to ensure adequate support.

- Fixings: All components securely fixed.

# Setting posts in concrete:

- Standard: To BS 8500-2.

- [Mix: Designated concrete not less than GEN1 or Standard prescribed concrete not less than ST2.]

- [Alternative mix for small quantities: 50kg Portland cement to150kg fine aggregate to 250kg, 20mm nominal maximum size coarse aggregate, medium workability.]

- [Admixtures: Do not use].

- Holes: Excavate neatly and with vertical sides.

- Filling: Position post/strut and fill hole with concrete to not less than the specified depth, well rammed as filling proceeds and consolidated.

- [Backfilling of holes not completely filled with concrete: Excavated material, well rammed and consolidated.

# Completion:

# Cleaning:

- General: Leave the works in a clean, tidy condition.

- Surfaces: Clean immediately before handover.

# Fixings:

- All components: Tighten.

- Timing: Before handover.