# Manufacturer: Jacksons Fencing

# Web: www.jacksons-security.co.uk

# Tel: +44 (0)1233 750393 | 0800 953 3699

# Product Reference: Jakoustic® Class 3 LPS 1175 C5 (SR3) /F1/G1

# LPS 1175 certified acoustic security barrier with up to 28dB* reduction in noise

# Certification:

# LPS 1175 C5 (SR3), F1, G1

# Guarantee:

25 year service life guarantee against rust and manufacturing defects

25 year Jakcure® guarantee against rot, insect attack and manufacturing defects

# Fencing & Posts:

# 2500mm High, Post Length 3250mm (2400mm Post Centres)

# 2750mm High, Post Length 2800mm (2400mm Post Centres)

# 3000mm High, Post Length 3300mm (2400mm Post Centres)

# 3250mm High, Post Length 3800mm (2400mm Post Centres)

# * Delete above as required

# Boards:

# Double layer of Jakcure pressure treated 34mm x 143mm x 4.8m (planed) boards with unique deep interlocking ‘v’ tongue and groove joint. For G1 rating a mesh layer is sandwiched between boards.

# Posts:

# Universal steel I beams

# Fixings:

# Special fixings attach the acoustic boards to the posts

# Finish / Colour:

[I beam posts supplied galvanised to BS EN 1461 as standard. Timber boards kiln-dried and Jakcure® pressure treated]

[I beam posts galvanised to BS EN 1461 then polyester powder coated with architectural Akzo Nobel Interpon grade powder in [Black RAL 9005]. [Green RAL 6005].  [White RAL 9010]. [Blue RAL 5010]. [Blue BS 20-C-40]. [Brown RAL 8017]. [Yellow RAL 1018]. [Grey RAL 7012]. [Red RAL 3020].]

For installations within 500m of salt water or estuary [Marine coated Insert RAL/BS]

Installation

# In accordance to manufacturer’s instructions

Accessories:

[None]. [Barbed wire topping]. [Razor wire topping]. [Integrated Jaktronic Perimeter Intruder Detection System]. [Gate post adapted for CCTV camera mounting] [Rota spike topping]

NOTES:

For more information please contact Jacksons Fencing. We reserve the right to alter specification within this document at any time.