# Manufacturer: Jacksons Fencing

# Web: www.jacksons-security.co.uk

# Tel: +44 (0)1233 750393 | 0800 953 3699

# Product Reference: UHS Heavy SR3 Fencing

# LPS 1175 SR3 certified double layer mesh fencing

# Certification:

# LPS 1175 SR3

# Guarantee:

25 year service life guarantee against rust and manufacturing defects

# Fencing & Posts:

# 2415mm High, Post Dimensions 80 x 80mm, minimum 2505mm Post Centres

# 3031mm High, Post Dimensions 80 x 80mm, minimum 2505mm Post Centres

# *Delete above as required

# *This product is LPS 1175 SR3 certified at the height and specification stated

# *Please note: Generic post sections and lengths may need to be altered to meet site specific location forces or added requirements.

# Mesh:

# Double skin 50 x 28mm mesh overlapped and bolted together with clamp bars and anti-tamper security fixings, giving 10.5 x 20.5 larger apertures and 6.5 x 20.5 smaller apertures. Pre galv.

# Panels:

# Must be buried or bolted down.

# Posts:

# Posts are hot dip galvanised inside and out to BS EN 1461 as standard and supplied with flat post caps in black polypropylene.

# Post dimension and top to be designed to [carry barbed wire / razor wire / other / security topping].

# Over length set in concrete as standard

# Base plated to bolt down onto concrete

# * Delete above post options as required

# Fixings:

# Panels are overlapped and secured with clamp bars and tamper proof fixings. Panels overlapped at posts with clamp bars and tamper proof fixings.

# Finish / Colour:

Panels and posts supplied galvanised inside and out to BS EN 1461 as standard.

Panels and posts supplied galvanised inside and out to BS EN 1461 as standard. Panels and posts polyester powder coated with architectural Akzo Nobel Interpon grade powder in [Black RAL 9005]. [Green RAL 6005].  [White RAL 9010]. [Blue RAL 5010]. [Blue BS 20-C-40]. [Brown RAL 8017]. [Yellow RAL 1018]. [Grey RAL 7012]. [Red RAL 3020]. Or special [Insert RAL/BS].

Marine coating required as installation is within 500m of salt water or estuary.

* Delete above as required

Setting Posts

# Holes: Excavate neatly and with vertical sides.

# Setting posts as standard in concrete: To BS 8500-2.

# [Mix: Designated concrete not less than GEN1 or Standard prescribed concrete not less than ST2.]  [Alternative mix for small quantities: 50kg Portland cement to150kg fine aggregate to 250kg, 20mm nominal maximum size coarse aggregate, medium workability.] [Admixtures: Do not use]

# Filling: Position post/strut and fill hole with concrete to not less than the specified depth, well rammed as filling proceeds and consolidated.  Backfilling of holes not completely filled with concrete: Excavated material, well rammed and consolidated.

NOTES:

For more information please contact Jacksons Fencing. We reserve the right to alter specification within this document at any time.